ENHANCED ISLAMABAD WITH INDUSTRIAL DEVICES

Additional Specialized Devices for Islamabad (DR Site)

1. Government/Emergency Communication Devices

| Device Type | Cisco PT Selection | Quantity | Purpose | Connection |
| --- | --- | --- | --- | --- |
| IP Phone | End Devices > IP Phone (7960) | 3 | Emergency hotlines | Connect to ISB-SW2/SW3 |
| Wireless Access Point | Network Devices > Wireless Devices > Access Point-PT | 2 | Mobile government staff | Connect to ISB-SW1 |
| Tablet | End Devices > Tablet-PT | 2 | Mobile emergency response | Wireless to AP |
| Laptop | End Devices > Laptop-PT | 2 | Field officers | Wireless to AP |
| Security Camera | End Devices > IP Camera | 2 | Facility monitoring | Connect to ISB-SW3 VLAN 12 |
| Printer | End Devices > Printer | 1 | Government documents | Connect to ISB-SW2 VLAN 10 |

IP Addressing for Additional Devices

| Device | IP Address | VLAN | Purpose |
| --- | --- | --- | --- |
| IP-Phone-1 | 10.1.10.20 | 10 | Management Hotline |
| IP-Phone-2 | 10.1.20.20 | 11 | Government Hotline |
| IP-Phone-3 | 10.1.30.20 | 12 | Emergency Hotline |
| Wireless-AP-1 | 10.1.10.30 | 10 | Management Area WiFi |
| Wireless-AP-2 | 10.1.20.30 | 11 | Government Area WiFi |
| Tablet-1 | 10.1.10.40 (DHCP) | 10 | Mobile Admin |
| Tablet-2 | 10.1.20.40 (DHCP) | 11 | Mobile Gov Officer |
| Laptop-1 | 10.1.10.41 (DHCP) | 10 | Field Officer 1 |
| Laptop-2 | 10.1.30.30 (DHCP) | 12 | Emergency Response |
| IP-Camera-1 | 10.1.30.40 | 12 | Entrance Camera |
| IP-Camera-2 | 10.1.30.41 | 12 | Server Room Camera |
| Printer-1 | 10.1.10.50 | 10 | Admin Printer |

Updated Islamabad Topology with Industrial Devices

To Peshawar

↑

Serial0/0/1

|

┌────────────────────┴────────────────────┐

│         ISB-R1 (Main Router)            │

│  Select: Network Devices > Routers >    │

│          Router-PT (2911 or 2901)       │

└────────────┬────────────────────────────┘

│

┌────────────┴────────────────────────────┐

│         ISB-R2 (Backup Router)          │

│  Select: Network Devices > Routers >    │

│          Router-PT (2911)               │

└────────────┬────────────────────────────┘

│

┌────────────┴────────────────────────────┐

│       ISB-SW1 (Core Switch)             │

│  Select: Network Devices > Switches >   │

│          Switch-PT (2960-24TT)          │

└───┬─────────────┬──────────────┬─────────┘

│             │              │

────┴───      ────┴───       ────┴───

ISB-SW2       ISB-SW3        Servers

(2960-24TT)   (2960-24TT)

│             │              │

────┴───      ────┴───       ────┴───

VLAN 10:      VLAN 11,12,13:  VLAN 100:

-3 PCs        -9 PCs          -DR-Server

-1 Printer    -2 Cameras      -DNS-Server

-3 IP Phones  -1 Laptop

-1 WiFi AP    -1 WiFi AP      VLAN 50:

-2 Tablets                    -2 IoT Sensors

Step-by-Step: Adding Industrial Devices to Islamabad

STEP 12: Add IP Phones

Where to find: End Devices > End Devices > IP Phone (7960)

Placement: Place 3 IP Phones

Configuration for IP-Phone-1 (Management):

- Click on IP Phone

- Physical Tab > Power off

- Remove existing module, add module: PT-PHONE-NM-1W

- Power on

- Desktop > IP Configuration

  - IP: 10.1.10.20

  - Mask: 255.255.255.0

  - Gateway: 10.1.10.1

  - DNS: 10.1.100.101

- Connect to ISB-SW2 Fa0/4 (VLAN 10)

For IP-Phone-2: 10.1.20.20, connect to ISB-SW3 Fa0/10 (VLAN 11) For IP-Phone-3: 10.1.30.20, connect to ISB-SW3 Fa0/11 (VLAN 12)

Update Switch Config:

! On ISB-SW2

interface FastEthernet0/4

description IP-Phone Management

switchport mode access

switchport access vlan 10

exit

! On ISB-SW3

interface FastEthernet0/10

description IP-Phone Government

switchport mode access

switchport access vlan 11

exit

interface FastEthernet0/11

description IP-Phone Emergency

switchport mode access

switchport access vlan 12

exit

STEP 13: Add Wireless Network

Where to find: Network Devices > Wireless Devices > AccessPoint-PT

Place 2 Access Points

Wireless-AP-1 Configuration (Management Area):

- Click AP

- GUI Tab > Setup

  - SSID: ISB-Management

  - Authentication: WPA2-PSK

  - PSK Pass Phrase: admin123

- Config Tab > Port 1

  - IP: 10.1.10.30

  - Mask: 255.255.255.0

  - Gateway: 10.1.10.1

- Connect to ISB-SW2 Fa0/5 (VLAN 10)

Wireless-AP-2 Configuration (Government Area):

- SSID: ISB-Government

- Authentication: WPA2-PSK

- PSK: gov123

- IP: 10.1.20.30, connect to ISB-SW3 Fa0/12 (VLAN 11)

Update Switch Config:

! On ISB-SW2

interface FastEthernet0/5

description Wireless-AP Management

switchport mode access

switchport access vlan 10

exit

! On ISB-SW3

interface FastEthernet0/12

description Wireless-AP Government

switchport mode access

switchport access vlan 11

exit

STEP 14: Add Mobile Devices (Tablets & Laptops)

Tablets - Where to find: End Devices > End Devices > Tablet-PT

Place 2 Tablets

Tablet-1 Configuration:

- Click Tablet

- Desktop > Config > Wireless0

  - SSID: ISB-Management

  - Authentication: WPA2-PSK

  - PSK: admin123

- Desktop > IP Configuration

  - DHCP (will get IP from 10.1.10.x range)

Tablet-2 Configuration:

- Connect to SSID: ISB-Government with PSK: gov123

- DHCP enabled

Laptops - Where to find: End Devices > End Devices > Laptop-PT

Place 2 Laptops

Laptop-1 Configuration:

- Physical Tab: Check if has wireless module (if not, power off, add WPC300N)

- Desktop > Config > Wireless0

  - SSID: ISB-Management

  - PSK: admin123

- DHCP enabled

Laptop-2 Configuration:

- Physical connection to ISB-SW3 Fa0/13 (VLAN 12 - Emergency)

- IP: 10.1.30.30 or DHCP

! On ISB-SW3

interface FastEthernet0/13

description Emergency Laptop

switchport mode access

switchport access vlan 12

exit

STEP 15: Add Security Cameras

Where to find: End Devices > End Devices > IP Camera

Place 2 IP Cameras

IP-Camera-1 Configuration (Entrance):

- Click Camera

- Config Tab > Settings

  - IP: 10.1.30.40

  - Mask: 255.255.255.0

  - Gateway: 10.1.30.1

  - DNS: 10.1.100.101

- Connect to ISB-SW3 Fa0/14 (VLAN 12 - Emergency/Security)

IP-Camera-2 Configuration (Server Room):

- IP: 10.1.30.41

- Connect to ISB-SW3 Fa0/15 (VLAN 12)

! On ISB-SW3

interface range FastEthernet0/14-15

description Security Cameras

switchport mode access

switchport access vlan 12

exit

STEP 16: Add Printer

Where to find: End Devices > End Devices > Printer-PT

Place 1 Printer

Printer-1 Configuration (Admin Printer):

- Click Printer

- Config Tab > FastEthernet0

  - IP: 10.1.10.50

  - Mask: 255.255.255.0

  - Gateway: 10.1.10.1

- Connect to ISB-SW2 Fa0/6 (VLAN 10)

! On ISB-SW2

interface FastEthernet0/6

description Admin Printer

switchport mode access

switchport access vlan 10

exit

UPDATED ISLAMABAD DEVICE COUNT

| Category | Quantity | Details |
| --- | --- | --- |
| Routers | 2 | ISB-R1, ISB-R2 (2911) |
| Switches | 3 | ISB-SW1, SW2, SW3 (2960) |
| PCs | 12 | Desktop computers |
| Servers | 2 | DR-Server, DNS-Server |
| IP Phones | 3 | VoIP Emergency Lines |
| Wireless APs | 2 | WiFi Coverage |
| Tablets | 2 | Mobile devices |
| Laptops | 2 | Field officers |
| IP Cameras | 2 | Security surveillance |
| Printers | 1 | Document printing |
| IoT Sensors | 2 | Temperature/Power monitoring |
| TOTAL | 31 devices | Professional setup |

NOW LET'S DO LAHORE WITH INDUSTRIAL DEVICES

LAHORE (Primary Data Center) - Enhanced Design

Additional Industrial/Specialized Devices for Lahore

| Device Type | Cisco PT Selection | Quantity | Purpose |
| --- | --- | --- | --- |
| Server Rack | End Devices > Server-PT | 3 | Primary DC, Web Server, Database |
| IP Phones | End Devices > IP Phone 7960 | 4 | IT Support, Finance, Healthcare |
| Wireless APs | Wireless Devices > AccessPoint-PT | 3 | Enterprise WiFi |
| Laptops | End Devices > Laptop-PT | 3 | Mobile IT staff |
| Tablets | End Devices > Tablet-PT | 2 | Healthcare mobile units |
| IP Cameras | End Devices > IP Camera | 3 | Datacenter security |
| Printers | End Devices > Printer-PT | 2 | IT + Finance printers |
| Smart TV/Display | End Devices > Smart Device | 1 | Monitoring dashboard |
| Smartphone | End Devices > Smartphone | 2 | Admin mobile access |

LAHORE - Complete IP Scheme with Industrial Devices

Routers

| Device | Interface | IP Address | Subnet Mask | Description |
| --- | --- | --- | --- | --- |
| LHR-R1 | Gig0/0 | 10.2.0.1 | 255.255.255.0 | LAN Gateway |
| LHR-R1 | Gig0/1 | 192.168.2.5 | 255.255.255.252 | Link to LHR-R2 |
| LHR-R1 | Serial0/0/0 | 192.168.1.2 | 255.255.255.252 | WAN to Islamabad |
| LHR-R1 | Serial0/0/1 | 192.168.1.5 | 255.255.255.252 | WAN to Karachi |
| LHR-R2 | Gig0/0 | 10.2.0.2 | 255.255.255.0 | LAN Gateway |
| LHR-R2 | Gig0/1 | 192.168.2.6 | 255.255.255.252 | Link to LHR-R1 |

Switches

| Device | Management IP | Purpose |
| --- | --- | --- |
| LHR-SW1 | 10.2.0.10 | Core Switch |
| LHR-SW2 | 10.2.0.11 | Distribution - Management/IT |
| LHR-SW3 | 10.2.0.12 | Distribution - Finance/Healthcare |

VLANs in Lahore

| VLAN ID | Name | Network | Gateway | Devices |
| --- | --- | --- | --- | --- |
| 20 | Management | 10.2.10.0/24 | 10.2.10.1 | 3 PCs + 1 Phone + 1 Printer |
| 21 | Finance | 10.2.20.0/24 | 10.2.20.1 | 4 PCs + 1 Phone + 1 Printer |
| 22 | IT-Admin | 10.2.30.0/24 | 10.2.30.1 | 4 PCs + 1 Phone + 3 Laptops |
| 23 | Healthcare | 10.2.40.0/24 | 10.2.40.1 | 4 PCs + 1 Phone + 2 Tablets |
| 100 | Servers | 10.2.100.0/24 | 10.2.100.1 | 3 Servers |
| 50 | IoT/Security | 10.2.50.0/24 | 10.2.50.1 | Cameras, Sensors |
| 60 | WiFi-Guest | 10.2.60.0/24 | 10.2.60.1 | Guest devices |

LAHORE - Complete Device List with IPs

15 Desktop PCs

| PC Name | VLAN | IP Address | Gateway | DNS |
| --- | --- | --- | --- | --- |
| LHR-PC1 | 20 | 10.2.10.10 | 10.2.10.1 | 10.2.100.102 |
| LHR-PC2 | 20 | 10.2.10.11 | 10.2.10.1 | 10.2.100.102 |
| LHR-PC3 | 20 | 10.2.10.12 | 10.2.10.1 | 10.2.100.102 |
| LHR-PC4 | 21 | 10.2.20.10 | 10.2.20.1 | 10.2.100.102 |
| LHR-PC5 | 21 | 10.2.20.11 | 10.2.20.1 | 10.2.100.102 |
| LHR-PC6 | 21 | 10.2.20.12 | 10.2.20.1 | 10.2.100.102 |
| LHR-PC7 | 21 | 10.2.20.13 | 10.2.20.1 | 10.2.100.102 |
| LHR-PC8 | 22 | 10.2.30.10 | 10.2.30.1 | 10.2.100.102 |
| LHR-PC9 | 22 | 10.2.30.11 | 10.2.30.1 | 10.2.100.102 |
| LHR-PC10 | 22 | 10.2.30.12 | 10.2.30.1 | 10.2.100.102 |
| LHR-PC11 | 22 | 10.2.30.13 | 10.2.30.1 | 10.2.100.102 |
| LHR-PC12 | 23 | 10.2.40.10 | 10.2.40.1 | 10.2.100.102 |
| LHR-PC13 | 23 | 10.2.40.11 | 10.2.40.1 | 10.2.100.102 |
| LHR-PC14 | 23 | 10.2.40.12 | 10.2.40.1 | 10.2.100.102 |
| LHR-PC15 | 23 | 10.2.40.13 | 10.2.40.1 | 10.2.100.102 |

Servers

| Server | IP | Services |
| --- | --- | --- |
| Primary-DC-Server | 10.2.100.100 | HTTP, FTP, DHCP, Database |
| Web-Server | 10.2.100.101 | HTTP, HTTPS |
| DNS-DHCP-Server | 10.2.100.102 | DNS, DHCP |

Industrial Devices

| Device | VLAN | IP Address | Purpose |
| --- | --- | --- | --- |
| IP-Phone-1 | 20 | 10.2.10.20 | Management Desk |
| IP-Phone-2 | 21 | 10.2.20.20 | Finance Desk |
| IP-Phone-3 | 22 | 10.2.30.20 | IT Support |
| IP-Phone-4 | 23 | 10.2.40.20 | Healthcare Reception |
| WiFi-AP-1 | 20 | 10.2.10.30 | Management WiFi |
| WiFi-AP-2 | 22 | 10.2.30.30 | IT WiFi |
| WiFi-AP-3 | 60 | 10.2.60.1 | Guest WiFi |
| Laptop-1 | 22 | DHCP | IT Staff Mobile |
| Laptop-2 | 22 | DHCP | IT Staff Mobile |
| Laptop-3 | 20 | DHCP | Management Mobile |
| Tablet-1 | 23 | DHCP | Healthcare Mobile |
| Tablet-2 | 23 | DHCP | Healthcare Mobile |
| IP-Camera-1 | 50 | 10.2.50.10 | DC Entrance |
| IP-Camera-2 | 50 | 10.2.50.11 | Server Room |
| IP-Camera-3 | 50 | 10.2.50.12 | Finance Area |
| Printer-1 | 22 | 10.2.30.50 | IT Printer |
| Printer-2 | 21 | 10.2.20.50 | Finance Printer |
| Smart-Display | 22 | 10.2.30.60 | Network Monitor |
| Smartphone-1 | 60 | DHCP | Admin Mobile |
| Smartphone-2 | 60 | DHCP | Manager Mobile |
| IoT-Sensor-1 | 50 | 10.2.50.20 | Temperature |
| IoT-Sensor-2 | 50 | 10.2.50.21 | Humidity |

LAHORE Physical Topology Map

To Islamabad          To Karachi

↑                    ↑

Serial0/0/0          Serial0/0/1

|                    |

┌────────────────┴────────────────────┴───────┐

│         LHR-R1 (Primary Router)             │

│  Device: Router-PT 2911                     │

│  HSRP Priority: 110 (Active)                │

└────────────────┬────────────────────────────┘

│ Gig0/1 (Redundancy Link)

┌────────────────┴────────────────────────────┐

│         LHR-R2 (Backup Router)              │

│  Device: Router-PT 2911                     │

│  HSRP Priority: 100 (Standby)               │

└────────────────┬────────────────────────────┘

│ Trunk

┌────────────────┴────────────────────────────┐

│       LHR-SW1 (Core Switch)                 │

│  Device: Switch-PT 2960-24TT                │

│  Connected: Servers + Distribution          │

└───┬─────────────────┬────────────────┬──────┘

│                 │                │

────┴───          ────┴───         ────┴───

LHR-SW2           LHR-SW3          Servers

(2960-24TT)       (2960-24TT)      Subnet

│                 │                │

────┴───          ────┴───         ────┴───

VLAN 20,22:       VLAN 21,23:      -Primary DC

-3 PCs (Mgmt)     -8 PCs           -Web Server

-4 PCs (IT)       (4+4)            -DNS/DHCP

-1 Printer        -2 Tablets

-3 Laptops        -1 Printer       VLAN 50:

-2 Phones         -2 Phones        -3 Cameras

-2 WiFi APs       -1 WiFi AP       -2 IoT

-1 Display

-2 Smartphones

LAHORE - Step-by-Step Implementation

I'll continue with detailed configurations. Should I proceed with:

- Router configurations (LHR-R1, LHR-R2)

- Switch configurations (all 3 switches)

- PC configurations

- Server setups

- Industrial device configurations

Or would you like me to first provide the complete device lists for remaining cities (Karachi, Peshawar, Quetta, Multan) with their industrial devices, and then we do detailed configuration city by city?

Which approach do you prefer?